| Knowledge Test – Safe in the Underground Environment |  |  |
| --- | --- | --- |
| Candidate Name |  |  |
| Date |  |  |
| Mark |  | Out of 85 |
| Pass/ Fail (80% pass, twenty marks required) |  |  |

| Question 1 | When arriving for work, what PPE / RPE are you required to be wearing, and what PPE must you have available on your person? | 2 Marks |
| --- | --- | --- |
| Answer 1 | PPE / RPE Being Worn - (Answer Required) PPE / RPE Being Carried on the Person - (Answer Required) |  |

| Question 2 | What must you do if you believe that you (or others) may be under the influence of alcohol or any others substance? | 1 Mark |
| --- | --- | --- |
| Answer 2 | - (Answer Required) |  |

| Question 3 | Briefly outline the reason that you must register that you have entered the mine before proceeding below ground? | 2 Marks (1 each) |
| --- | --- | --- |
| Answer 3 | The answer must include - Provides a record of people that entered the mine (for emergency purposes) - Provides a record of how long a person has been below ground (missing persons) |  |

| Question 4 | Describe how you would register that you are entering the mine | 1 Mark |
| --- | --- | --- |
| Answer 4 | - (Answer Required) |  |

| Question 5 | What items are you not allowed to take underground (this may be termed contraband) | 2 Marks |
| --- | --- | --- |
| Answer 5 | - (Answer Required) |  |

| Question 6 | When thinking about risk and risk assessment, what is meant by the term ‘hazard?’ | 1 Mark |
| --- | --- | --- |
| Answer 6 | - Something that has the potential to cause harm |  |

| Question 7 | When thinking about risk and risk assessment, what is meant by the term ‘risk?’ | 1 Mark |
| --- | --- | --- |
| Answer 7 | - The chance or likelihood that the hazard will cause harm |  |

| Question 8 | Briefly outline the typical steps that should be taken when conducting a risk assessment? | 6 Marks (1 each) |
| --- | --- | --- |
| Answer 8 | The answer must mention 6 of these for full marks - The risk assessment may be completed as an individual or group - There should be a process that identifies the hazards which are present - Consider how/ who might be harmed - Consider the risk of that event/ hazard materialising - Consider the types of controls that need to be in place - Formally record the findings of the risk assessment - Update as required |  |

| Question 9 | Explain what is meant by the hierarchy of controls? | 1 Mark |
| --- | --- | --- |
| Answer 9 | - The answer should indicate that these are a series of steps that should be applied to eliminate or reduce the risk |  |

| Question 10 | From the hierarchy of controls identify typical steps that an employer should consider? | 4 Marks (1 each) |
| --- | --- | --- |
| Answer 10 | The candidate can have any four from the following list - Eliminate - Reduce - Isolate - Control - PPE - Behaviours |  |

| Question 11 | What is meant by a point of work risk | 2 Marks |
| --- | --- | --- |
| Answer 11 | The candidate should have an answer that incorporates the following points - Assessment conducted by the employee at the point at which the work is carried out - It should be used to allow the employee to take the time to consider the hazards, risk, and controls - Assist in visualising the controls - Embed a sense of responsibility and ownership |  |

| Question 12 | What is meant by the term method statement | 2 Marks |
| --- | --- | --- |
| Answer 12 | The candidate should have an answer that incorporates the following points - It is something that provides a structured and defined way for completing the task (A step by step process) - It should incorporate the key controls identified within the risk assessment |  |

| Question 13 | Identify five general or health hazards that may be present within the underground mine environment | 5 Marks (1 each) |
| --- | --- | --- |
| Answer 13 | The candidate should have five of the following: - Trips and slips - Electricity - High Pressure Systems - Stored Energy - Vehicle/ Vehicle or Vehicle/ Pedestrian interactions - Noise - Vibration - Airborne Dust - Diesel exhaust emissions - Heat |  |

| Question 14 | What is meant by the term ‘major mining hazards’ | 2 Marks |
| --- | --- | --- |
| Answer 14 | The candidate should have an answer that incorporates the following points - Where an event has the potential or escalation potential to cause multiple fatalities, multiple casualties, catastrophic damage or adversely impact the image/ profitability of the organisation |  |

| Question 15 | Identify four major mining hazards that could be present in the underground mining environment? | 4 Marks (1 Each) |
| --- | --- | --- |
| Answer 15 | The candidate can have any of the following from the list: - Fire - Explosion - Ground Control - Inrush - Mass Transport (which would include shafts and winders) |  |

| Question 16 | Identify four other hazards that may be present in the underground environment that may have the ability to cause serious or fatal injury | 4 Marks (1 each) |
| --- | --- | --- |
| Answer 16 | The candidate can have any of the following from the list: - Falling objects (from height) - Falls from height - High pressure systems - Asphyxiation/ Toxic/ Noxious Gasses - Electrical Systems - Use of explosives - Mobile Plant - Lifting Operations - Conveyor or mineral transport systems - Long term exposure to substances hazardous to health |  |

| Question 17 | Identify the general duties of employees required by the Health and Safety at Work Act | 3 Marks |
| --- | --- | --- |
| Answer 17 | The candidate should identify the following key points to receive all the marks: - Take care of your self - Take care of others - Co-operate with the employer on matters of health and safety - Follow the training that has been provided when using equipment provided by the employer - Let the employer know if you think that the work is putting anyone’s health or safety at risk |  |

| Question 18 | What is the main piece of legislation that is used to regulate underground mining activities in the UK | 1 Mark |
| --- | --- | --- |
| Answer 18 | - The Mines Regulations 2014 |  |

| Question 19 | Why is mine ventilation important and what it is used to achieve | 2 Marks |
| --- | --- | --- |
| Answer 19 | Looking for the candidate to indicate that: - Ventilation is used to - Provide cooling - Dilute noxious/ toxic/ explosive gases - Dilute dust or other things that may be harmful - Maintain the correct levels of oxygen (provide fresh air) - Import that it is adequately maintained as without it an unsafe, unhealthy working environment would be created very quickly |  |

| Question 20 | Why is it important that you close any doors or regulators that you pass through | 1 Mark |
| --- | --- | --- |
| Answer 20 | Looking for the candidate to indicate that, if doors are left open then there is potential to short circuit the air within the rest of the mine, which means those inbye of that location are left without fresh air |  |

| Question 21 | Identify four things that you should never do when it comes to maintaining adequate mine ventilation | 4 Marks (1 Each) |
| --- | --- | --- |
| Answer 21 | Looking for the candidate to identify any four from the following list: - Allow air doors to remain open (takes air away from people further into the mine) - Stop a fan that is ventilating any area of the mine, unless instructed or there is immediate danger - Make holes in any ventilation ducts - Do not restart a fan which has been stopped without being given permission to do so - Pass any barriers that are in place, unless given permission to do so - Make any changes to a ventilation system |  |

| Question 22 | What are the main categories of gas that may be found underground | 3 Marks (1 Each) |
| --- | --- | --- |
| Answer 22 | The candidate gets one mark for each of the following: - Explosive (or flammable) - Toxic - Noxious (poisonous) |  |

| Question 23 | Identify by percentage the main constituents of a normal mine atmosphere? | 3 Marks (1 Each) |
| --- | --- | --- |
| Answer 23 | The candidate gets one mark for each of the following: - Nitrogen – 78% - Oxygen – 21% - Other – 1% |  |

| Question 24 | What is the occupational exposure limit for carbon monoxide for a period of 8 hours | 1 Mark |
| --- | --- | --- |
| Answer 24 | - 20PPM |  |

| Question 25 | What is the minimum permissible level of oxygen that is allowed underground in a mine? | 1 Mark |
| --- | --- | --- |
| Answer 25 | - 19% |  |

| Question 26 | Briefly outline how you would use a personal gas monitor to monitor gas in the mine environment? | 2 Marks |
| --- | --- | --- |
| Answer 26 | Within the answer we are looking for the candidate to identify: - Check out the monitor in fresh air at the surface - Gases have different densities and therefore there is a need to test at various levels - Need to consider cavities (if they exist) - That the gas detector needs to be placed into the gas - The types of gases that can be detected will be dependent on the sensors that are fitted |  |

| Question 27 | What actions would you take if your personal gas monitor were to alarm? | 2 Marks |
| --- | --- | --- |
| Answer 27 | Within the answer we are looking for the candidate to identify: - If possible, compare the result with a different monitor - If not possible withdraw to a different area and see if the reading changes - Make your supervisor or someone in authority aware of the situation - Withdraw to a safe area |  |

| Question 28 | For a fire to occur what three elements need to be present | 2 Marks |
| --- | --- | --- |
| Answer 28 | The candidate needs to identify all three for the marks: - Heat (Ignition source) - Fuel - Oxygen |  |

| Question 29 | What are the potential hazards associated with a fire occurring underground? | 2 Marks |
| --- | --- | --- |
| Answer 29 | The candidate needs to identify all for the marks: - Heat (Burns) - Gases (Carbon Monoxide/ Carbon Dioxide) [Toxic/ Noxious] - Reduced visibility - Reduced access / egress |  |

| Question 30 | If you are asked to fight a fire, what considerations should you have? | 4 Marks (1 Each) |
| --- | --- | --- |
| Answer 30 | The candidate needs to identify all for the marks: - Is the fire too big? - I am in a position where the air is safe to breathe - Is the environment too hot or smoky? - Is there a safe evacuation path? |  |

| Question 31 | Outline the process for raising the alarm in an emergency underground? | 2 Marks |
| --- | --- | --- |
| Answer 31 | - Answer Required |  |

| Question 32 | If you have an accident which requires first aid treatment, what information will you be expected to provide? | 2 Marks |
| --- | --- | --- |
| Answer 32 | The candidate should have an answer which captures the following points: - The nature of your injuries - Any assistance that you require - Are you leaving your place of work / exiting the mine - The location of the accident - How the accident occurred - When that accident occurred |  |

| Question 33 | How would you identify the locations of first aid stations within the workplace? | 1 Mark |
| --- | --- | --- |
| Answer 33 | - Answer Required |  |

| Question 34 | If you must raise an alarm, what information would you be expected to supply? | 2 Marks |
| --- | --- | --- |
| Answer 34 | The candidate should have an answer which captures the following points: - What is the nature of the emergency? - What has happened? - How many people are affected? - When has it occurred? - Where you are - Anything else that those responding to the emergency might find useful |  |

| Question 35 | If you must wear a self-rescuer/ escape set, when would you take it off? | 2 Marks |
| --- | --- | --- |
| Answer 35 | The candidate should have an answer which captures the following points: - When you reach a place of safety (this may be the surface) - When instructed to do so by someone in authority |  |

| Question 36 | Outline the values of your organisation? | 2 Marks |
| --- | --- | --- |
| Answer 36 | - Answer Required |  |

| Question 37 | Outline the behaviours your organisation expects to bring these values to life? | 3 Marks |
| --- | --- | --- |
| Answer 37 | - Answer Required |  |

| Question 38 | What is the potential impact if people do not behave correctly in the workplace? | 2 Marks |
| --- | --- | --- |
| Answer 38 | Looking for the candidate to identify that: - Behaviours can manifest themselves in different ways - Poor behaviour has the potential to impact the safety, moral and the culture of the organisation - Failure to address this may lead to incidents, reduced performance all of which has a negative impact on the reputation and viability of the business |  |

| Signed (Examiner) |  |
| --- | --- |
| Print Name (Examiner) |  |